TP-76500 Support for MQ V 9.2

Changes being made in ANTZ.RIT.V1020.TP76500.WMQ.SRC

From Rich Harran (Richard Harran <RHARRAN@uk.ibm.com>):

Yes, the V9.2 queue manager has been changed to prevent that SSAR access. Earlier versions are likely to have the same change backported to them.

It's not safe for you to set the AX for the ally unless you control that whole address-space, as that will apply outside of your authorized routine.

In general, you should set-up an EAX for your PC routine, and use that to get access to the MQ address-space in that routine only (I think you may already have an EAX for your PC).  This technique is described here: https://www.ibm.com/docs/en/zos/2.2.0?topic=registers-eax-authority-address-space, although that assumes you already have a service running in the MQ address-space, so you'd have to schedule an SRB instead. Your PC can then add a private alet to the QM as required. 

If it really is just the chinit, that already has a public PASN ALET to the QM.  This ALET is available in a system level name token with name starting 'CSQXQMAL', then with the queue manager name appended twice (eg 'CSQXQMALCSQ1CSQ1').  The ALET is the first four bytes of the token value. If you use that, you don't have to set any extra authority.

Additional note from Rich:

Hi Mark,

Running with the right EAX doesn't give you the corresponding authority to issue SSAR, just the authority to use an ACCESS=PRIVATE ALET that is covered by that entry in the AT.

You will have to set the ALET in the register(s) that you use to access the QM address space, rather than using an ALET of 1 (=secondary ASN/ASID) and using SSAR to set the secondary ASID to the QM. So starting from the old code I have, you might change to something like:

L     R2,QMGRALET
         DROP  R10

SAC   512                     put us in AR mode
* R6 -> name and does not change
* R8 -> name and gets updated
         SAR   R6,R2                   set access reg 6 from gr 2
         SAR   R8,R2                   set access reg 8 from gr 2
         J     L_FINDCONT

for example.

I believe you can also avoid some complications by using a special entry-table entry number of 1, corresponding to an EAX of 1. This entry is set to be authorised in all address-spaces, so it you run your PC with an EAX of 1, that gives your code access to use a private ALET to any address-space and you then don't have to mess with the AT in the QM address-space, which can be problematic.

Also make sure that you don't keep adding ALETs without deleting them, as they go in fixed-size tables.  You may find that there's a noticable performance hit if you add and delete on every intercept call, so it might be better to keep a PASN ALET for the address-space, or a DU-AL ALET for the TCB if you have the infrastructure to store these, find them again and clean them up on QM disconnect.

See figure 2 on this page to see how it is currently working: https://www.ibm.com/docs/en/zos/2.3.0?topic=environment-address-space-authorization

Setting up the EAX:

I need to issue an AXRES to obtain the EAX: https://www.ibm.com/docs/en/zos/2.1.0?topic=space-setting-eax-value

This page mentions using the value returned from AXRES as an EAX through the ETDEF, ETCRE, and ETCON macros: https://www.ibm.com/docs/en/zos/2.3.0?topic=dyn-axres-reserve-authorization-index

- ETDEF – Create an entry table descriptor. In other words, it builds parms used by ETCRE to build an entry table.

  - Invoked in HRVMASM. Then SRB is scheduled in queue manager. The SRB creates the entry table, so the entry table resides in the qmgr’s address space.

  - EAX= is not specified. When not specified, the PC routine has the same EAX as the issuer of the PC instruction. Should we do an AXRES in HRVMASM, and then use the returned value on the ETDEF? I think this would associate the EAX with the PC.

- ETCRE – Create entry table

  - Invoked in SRB (HRVMMQF). Did not find a reference to EAX in the doc for this macro.

- ETCON – Connect entry table

  - Invoked in SRB (HRVMMQF). Did not find a reference to EAX in the doc for this macro.

From http://www.kcats.org/csci/567/ho/pcrtn.pdf, an entry table is an array of PC routine definitions. An entry table entry, is an individual PC routine definition, and an entry table descriptor describes the array of PC routines. ETDEF is used to define Entry Tables, Entry Table Entries, and the Entry Table Descriptor.

Authorizing the EAX to do SSAR

From Rich’s note: “Your PC can then add a private alet to the QM as required.”

SRB that is scheduled by HRVMASM in the qmgr as needs to do the ATSET

The PC needs to issue ALESERV: https://www.ibm.com/docs/en/zos/2.1.0?topic=space-setting-eax-value with ACCESS=PRIVATE and CHKEAS=YES (which is the default)

Background:

| Address space that creates PC |  | Accessing address space - Address space where our PC runs. This could be xxxxCHIN or a batch job | Target address space – Queue manager |
| --- | --- | --- | --- |
| HRVMSASM | Queue manager (via SRB) |  |  |
|  | Issue the AXRES macro to reserve an AX to be placed in the entry table descriptor (ETD) as the EAX value of the PC routine. |  |  |
|  | Place the AX in a common area so that a program in the target address space can obtain it and set its AT accordingly. |  |  |
|  | Issue the ETDEF macro to build the PC routine's ETD. On the EAX parameter, code the address of the AX value that is to be the EAX of the PC routine. |  |  |
|  | Issue the ETCRE macro to build the entry table. |  |  |
|  | Issue the LXRES macro to reserve a linkage index (LX) in the linkage table. |  |  |
|  | Issue the ETCON macro to connect the entry table to the linkage table entry. |  |  |
|  |  | While the PC routine is running, the caller has the EAX value that the EAX parameter on ETDEF defined. The PC routine can … Issue the ALESERV ADD macro to add an entry for the target address space to the access list. |  |

- Procedure for the accessing address space (https://www.ibm.com/docs/en/zos/2.1.0?topic=space-procedures-accessing-address)

  - The PC routine (HRVMMQF) should contain an “ALESERV ADD request for an address space and any code that manipulates data in the target address space.”

- what is a DU-AL and a PASN-AL? (https://www.ibm.com/docs/en/zos/2.1.0?topic=uar-access-lists)

  - DU-AL – dispatchable unit (a TCB or SRB) access list.

  - PASN-AL – primary address space access list

  - Programs add entries representing address spaces that they want to access

  - A single program can use both a DU-AL and an PASN-AL to establish addressability to other address spaces (https://www.ibm.com/docs/en/zos/2.1.0?topic=lists-comparison-pasn-al-du-al)

  - Establishing addressability to an address space in order to use an AR to reference data in that address space:

    - Must have authority to access data in the address space

    - Must have an access list entry that points to the address space

      - Use ALESERV ADD to create an access list entry. Need to supply the STOKEN for the address space being accessed, and ALESERV returns the ALET for the entry.

      - Access list entries can be either public or private. In our case, it will be private.

    - Must have the ALET that indexes that entry

To do:

- Update HRVMASM to create an EAX for the PC

  - AXRES to reserve an AX https://www.ibm.com/docs/en/zos/2.1.0?topic=dyn-axres-reserve-authorization-index  Added AXL, AXCOUNT, and PCEAX to PCDEF. Since there was some extra space at the end of PCDEF, there was no need to extend PCDEF.

    - MQV8 requires at least 1.13 of z/OS, so it’s safe to add OSREL=ZOSV1R6 to the SYSSTATE macro. This eliminates the need to provide a save area for registers on entry into AXRES.

  - Update the ETDEF to use the AX value by specifying EAX= on the ENTRY

    - DSAINIT contains an ETDEF and is copied into DSA in line 142. Why? Will my modification of ETDEF impact this?

      - DSAINIT (line 1098) is copied to INITIALIZED_DSA (part of DSA DSECT within HRVMSASM), which is where ETDESC resides.

      - ETDESC is later copied to dETDESC, which is part of PCDEF, which is the common block of storage shared between HRVMASM, the SRB, and HRVMMQF

      - What is lETDESC before and after changes?

        - Before: 9C – 70 = 2C

        - After: ????

    - After adding EAX=PCEAX to ETDEF, QM01RIT ends with RC8

      - Where should AXRES be done?

        - What actually creates the new PC and assigns the number to it?

          - HRVMSASM 624 – 639 points to WNEWPRH2 which was populated from APIEXITPC which was populated from LXVALUE after the scheduling of the SRB. LXVALUE was populated from the LXRES that was done in the SRB.

        - Since the LXRES is being done by the SRB, the ETCON must also be done there: “The linkage index and the entry table being connected must be owned by the same task (the cross memory resource owning task of the home address space).” - Description - IBM Documentation

        - Option 0 – AXRES in HRVMSASM, pass PCEAX to SRB in PCDEF, leave everything else as is. Changes made to HRVMMQF0 and HRVMSAS0.

          - HRVMSASM ended with RC6, breadcrumb ETCRE  Set in HRVMSASM 467. Next Breadcrumb was set in line 572, so error occurred between 467 and 572. The only jump to END_3 is in 550 after a bad rc from IEAMSCHD.

          - HRVMSTRT invokes debug() to print debug info including PCDEF. Not sure how it gets a copy of PCDEF, but I should update the PCDEF in RITUSERP.H to include the new EAX related fields.

          - R15 stored in FUNCRC. HRVMSTRT’s copy of FUNCRC contains 1C which means the SRB could not be scheduled or did not complete successfully. Additional info is saved in address pointed to by:

            - SYNCHCOMPADDR: 00000008 SRB ended abnormally

            - SYNCHCODEADDR: 00052000

            - SYNCHRSNADDR: 00000319 The extended authorization index (EAX) specified was not reserved by the home address space.

        - Option 1 – move AXRES to existing SRB. Changed made to HRVMMQF1 and HRVMSAS1

          - Need to add SRB for removing AX when queue manager and/or agent stops?

          - This also requires moving the ETDEF to the SRB which caused compile problems.

            - “ASMA033I Storage alignment for PCEAX unfavourable”

            - No active using for amodule which contains the entry point address for HRVMMQF.  saved to MQFEntryPoint in PCDEF, and replaced reference to aModule with MQFEntryPoint to resolve this.

            - Multiple undefined symbols for ET* fields.

        - Option 2 – move LXRES and ETCRE to HRVMSASM. Will still need SRB for issuing ATSET

        - Option 3 – create flag to determine function to be performed by SRB. Schedule SRB 3 times: first to issue AXRES, then to do LXRES/ETCON, and finally to do AXFRE.

          - Create flags within PCDEF - done

          - Add code within SRB to branch to appropriate section for function requested. - done

          - Add code to HRVMSASM to schedule SRB multiple times.

            - The following are referenced in the code I copied to just prior to the ETDEF in order to invoke the SRB to obtain the EAX:

              - XC    SCHED_RC1,SCHED_RC0  Don’t know why this is being done, but will leave it as is for now

              - XC    SCHED_RC1,SCHED_RC1  Don’t know why this is being done, but will leave it as is for now

              - XC    SCHED_RC1,SCHED_RC2  Don’t know why this is being done, but will leave it as is for now

              - XC    SCHED_RC1,SCHED_RC3 Don’t know why this is being done, but will leave it as is for now

              - R2 – contains aModule. aModule again loaded into R2 prior to ETDEF, so ok

              - A_ETCON – updated again just prior to second SRB call, so ok

              - R0 - ok

              - RITFLAGS+1 - ok

              - RITFLAGS+2 - ok

          - Add code to HRVMSTRT to handle AXRES (eAXRES 24) or AXFRE (eAXFRE 25) failures.

- Update HRVMMQF SRB to give SSAR access to the EAX

  - Obtain the EAX from a new field within PCDEF  What are the migration considerations? Do I need any safeguards in case the new HRVMMQF is used with the old HRVMASM or vice versa?

    - How does the SRB associate the ETDEF with the ETCRE?

      - ENV=STOKEN and TARGETSTOKEN are used to cause the SRB to execute in the qmgr

      - SYNC=YES – causes HRVMASM to suspend until the SRB has completed

      - PARM= I should add this to the PCDEF, but can I replace the QMGRASID field that is currently being used? Or is QMGRASID needed for other reasons?WXTOKEN – causes contents of WXTOKEN to be loaded into R1 when SRB receives control. WXTOKEN contains the address of the storage that was obtained by HRVMASM. This is mapped by PCDEF

      - ETCRE ENTRIES=DETDESC – DETDESC is a field in PCDEF

  - ATSET using the EAX as an input parameter – done  come back and review storage keys prior to and after updating SRBFLAG

- Update HRVMASM to save the qmgr’s STOKEN in PCDEF so that it can be used later by HRVMMQF’s ALESERV

  - QMGRASID is a field in PCDEF, which is set in HRVMSASM from SCOMASID. It is later used by HRVMMQF before doing the SSAR.

  - The STOKEN for the qmgr asid is available in HRVMASM’s MYSTOKEN.

- Update HRVMMQF to find the ALET for the QMGR

  - ALET SEARCH using the qmgr’s STOKEN to retrieve the ALET

  - ALESERV ADD to create an entry in the PC’s access list for accessing the qmgr. This returns the ALET - Can I do this only on the first pass and save the ALET in PCDEF? Is the table entry being added for the home as? If so, could each invocation of the PC be running in a different as? In that case, should I query the table first before doing the ALESERV to add the entry?

    - The SSAR is done in:

      - 1939 - F7_FindNextQueue – called from 389 (divert) and 872 (duplicate)

      - 2198 - L9_NotPUT1

      - 2292 – LA_FRRCOPY

    - Compile errors: ASMA034E Operand IHB0018J beyond active USING range by -5660 bytes
staticArea starts at 1A5C, IHB0018J at 440, 1A5C-440=161C (5660 decimal). Added LARL and USING statement just prior to and DROP statement just after ALESERV to resolve problem.

  - Place the ALET in the appropriate register prior to SSAR - ????? why did the old code place an ASID in R2 prior to the SSAR, and I will now be placing an ALET?

- Update HRVMASM to free the EAX when the agent is stopped  Don’t forget to redo this, now that I have switched to creating the EAX from the SRB.

  - AXFRE the EAX stored in PCDEF. Do this as part of END_3 to handle the case where the agent ends due to some unexpected error being caught.

    - Added code to free the EAX immediately following code that frees storage, however, it does not appear to be invoked when doing a normal stop.

    - HRVMSTRT is invoking HRVMSASM (ritsasm) with a parm of ‘P’. HRVMSASM jumps to label STOP.

    - Changed code to jump unconditionally to END_3 after doing AXRES, but it’s not happening. Is it because the PC has already been created, and my new AXRES is not being invoked? Backing up from SWAPOK, it looks to me like my code should getting invoked. Recompiled with larger PCDEF. This should have forced new PCDEF to be allocated, and I hoped it would force my code to be invoked and jump to END_3. Still no luck. Started reviewing from beginning of HRVMSASM:

      - Check the length of USERPARM, which is the block shared between the c programs and the assembly routines defined in RITCOPY. Jump to TERM_NOT_SS if there is a problem

      - Check for USRP string within USRPARM. Jump to TERM_NOT_SS, if it isn’t there.

      - Check authorization. If not authorized, jump to TERM_NOT_SS, else jump to GET_DSA. DSA will be used to store ETDEF

      - Get DSA storage. If successful, jump to DSAGETOK, else jump to NO_ReleaseDSA. NO_ReleaseDSA is in the same section of code as END_1, END_2, END_3, FREE_DSA, Q_END, and TERM_NOT_SS

      - 138-142 Initialize DSA.

      - 147-150 Jump to either STARTUP or STOP, depending on whether passed parm is S or P. If neither, assume unit test and then jump to END_3.

      - 171 Jump to GETENQ to enqueue on RITAPIXT to ensure it is single threaded, and then return. If successful, jump to LOCATEMOD, else jump to Q_END.

      - 183-200 Invoke CSVQUERY to see if HRVMMQF is loaded into the LPA. If not, jump to Q_END. If so, jump to EXIT_LOAD_OK.

      - 232-247 Call IEANTRT to see if there is a name/token pair for the agent. If not, jump to GET_NEW_PCD. If so, jump to HaveOld. If some other unexpected value from compere, jump to END_1.

      - 253-266 WXTOKEN should point to previously obtained storage for the “global area”. The global area is to be accessible by the SRB and with HRVMMQF. If it is 0, jump to END_1. If it is non-zero, jump to WXTNZ.

      - 268-274 Check whether the size of the global area has changed. If it has, jump to GET_NEW_PCD, else continue on to PCDLEN_OK, which then jumps to Use_Old_LX

        - 293-309 Obtain storage for PCDEF. If successful, jump to StorageOK1. If not, jump to END_2.

        - 311-334 At this point, R9 should point to the WXTOKEN for the original storage. If R9 is 0, jump to NODELETE, otherwise release the old storage.

        - 337-380 Copy key values to newly acquired PCDEF, and create a name/token pair pointing to it. If name/toke pair created successfully, jump to NameTokenAvail. If not, jump to END_2.

      - 383-405 Use_Old_LX: Jump to FIND_RMVT to find the qmgr’s rmvt, and return here. If successful, jump to FOUNDSSCT, else jump to END_3.

      - 407-420 FOUNDSSCT: invoke LOCASCB. If successful, jump to LOCASCBOK, else jump to END_3.

      - 421-463 LOCASCBOK: Do ETDEF, move ETDESC to dETDESC, and jump to EXETROK.

      - …

    - Problem was simply that HRVMSASM was linked statically with HRVMSTRT. Ran link job after compiling HRVMSASM, and I can now see my changes taking effect.

    - Now getting abend 201 when starting agent if I try to force early termination by setting bad return code in R15 after AXRES. Setting the bad return code in R15 in the function prior to the AXRES results in the agent ending and setting an RC 8 without producing the 201 abend. After this, I tried again with the bad return code set after the AXRES, and got a RC8 rather than abend 201.

    - I then uncommented the AXFRE, and now receive AbendS052 RC0803 with 0 in R4, Perhaps this is because I only reserved it, but never actually used it? Come back to this after including it on the ETDEF and test again.

  - AXFRE the EAX stored in PCDEF. Do this as part of code that is executed when HRVMSASM is called with the parm ‘P’.

    - 778 Call UNSWAP

    - 781 jump to END_3. This is the same routine that is used to clean up in the case of an unexpected end, so this work has already been done.

Debugging

- RIT332W RIT interception is inactive

- After last compile, not seeing ALESERV RC. Recompiled to do “About to do ALESERV” wto. Before starting recording in RIT, F QM01RIT,DIS ALL shows RIT recording is active. After starting recording in RIT, same command shows recording is inactive.

  - Are my wto’s causing problems by corrupting register contents?

  - Is the FRR routine getting control and creating a dump? If so, what can I find from the ump? What data does the FRR routine preserve for me?

  - Recreating latest problem, rit interception is active until I start recording. Then I get 6 sets of ALESERV messages, followed by “Refresh duplicate list successful”. Then 1 set of ALESERV messages followed by “refresh divert list successful”. Then recording is inactive. Got a dump from QM01MSTR. Recreated again and got a dump from QM01CHIN.

    - Dump shows 0D5 rc 21 – “An address space second table index (ASX) translation exception (program interruption code X'21') occurred. The value in the address space second table (AST) corresponding to the ASID is not valid. Either no such address space exists or the system had swapped it out.”

    - Error was at offset 17F8, which is the SSAR in L9_NotPUT1

    - R10 should contain the pointer to PCDEF: xxxx1850, so ETCON_RC is at 18CC and AXCOUNT is at xxxx1A00

    - 07C:	ETCON_RC: 00000000

    - 080:	ETCRE_RC: 00000000

    - 084:	LXRES_RC: 00000000

    - 154:       QMGRASID: 000000E6

    - 1B0:	AXCOUNT(H) 0001

    - 1B2:	PCEAX(H) 001B

    - 1B4:	SRBFLAG 02000000  perform LXRES and ETCON

    - 1B8:	AXRES_RC 00000000

    - 1BC:	AXFRE_RC FF000000

    - 1C0:	ATSET_RC 00000000

    - 1C4:	ALESERV_RC 00000000

    - 1C8:   	QMSTOKEN 00000398   0000001C

    - 1D0:	QMALET: 0001002C

    - 1D4:	DALESERV:

      - ALET:  0001002C

      - STOKEN:  00000398   0000001C

    - All RC fields 00 except for AXFRE_RC, which is as expected. Need to verify that I am making the right ALESERV call.

    - R2 should contain QMALET

      - R2: 00010085

      - QMALET: 0001002C

- Testing on 9.2:

  - After switching back to using the QMGRASID for the SSAR, received 0D7 RC25. This is the same as the original abend.

  - R10 contains 1D42A1C0

  - 154: QMGRASID: 0000005A

Hi Hank,

I hope you don’t mind a couple of general questions related to PC’s and cross memory services. I work with the HCL OneTest API product (also known as Rational Integration Tester), and we have an agent for MQ that no longer works as of MQ V9.2. Our agent consists of a program that starts it up and a PC that is invoked by MQ as it puts messages on queues. The PC needs access to storage within the queue manager, and until recently, was able to do it with an SSAR.

According to a contact at IBM, the MQ V9.2 queue manager was changed to prevent that SSAR access. His recommendation was to assign an EAX to our PC, and to add a private ALET to the queue manager. My background is that I have done a bit of development in a variety of languages, including s390 assembly. However, my assembly language skills are fair, and I have almost no experience with cross memory services. I have read quite a bit in the knowledge center, but I still suspect that I might be missing something basic.

Below is a description of the original flow of our agent. I inherited it from someone else, so there are some things I am not clear on. For instance, I’m not sure why the LXRES, ETCRE, and ETCON were implemented in an SRB in the queue manager address space.

After that, is a description of the flow of my modified agent. I added an AXRES to reserve an EAX. I did this by scheduling it in an SRB within the queue manager address space, since I believe it needs to be done in the same address space that executes the ETCRE. I then added the AX returned by AXRES to the EAX parm of the ETDEF to associate it with the PC. I added ATSET to the SRB that does the ETCRE to authorize SSAR for the EAX. Finally, I do an ALESERV in the PC.

I have been receiving either

Original agent flow

- Program A is run when the agent starts.

  - Obtains storage to be shared with the SRB and with the PC

  - Verifies that the module containing the PC has been loaded into lpa and obtains the entry point address.

  - Executes ETDEF

  - Schedules synchronously executed SRB in the MQ queue manager address space.

- SRB in the queue manager address space executes the following macros to set up the PC:

  - LXRES

  - ETCRE

  - ETCON

- The PC gets invoked by various programs that put messages to MQ queues.

  - Loads ASID of queue manager into R2

  - Issues SSAR R2

  - Issues SAR to load x’01’ into ARs, and access data in queue manager address space

Modified agent flow

- Program A is run when the agent starts.

  - Obtains storage to be shared with the SRB and with the PC

  - Verifies that the module containing the PC has been loaded into lpa and obtains the entry point address.

  - Schedules synchronously executed SRB in the MQ queue manager address space to perform AXRES, saving AXRES in shared storage field PCEAX.

  - Executes ETDEF with EAX=PCEAX

  - Schedules synchronously executed SRB in the MQ queue manager address space.

- SRB in the queue manager address space executes the following macros to set up the PC:

  - LXRES

  - ETCRE

  - ETCON

  - ATSET AX=PCEAX,PT=NO,SSAR=YES

- The PC gets invoked by various programs that put messages to MQ queues.

  - ALESERV ADD,ALET=QMALET,STOKEN=QMSTOKEN,CHKEAX=YES,     ACCESS=PRIVATE,MF=(E,DALESERV)

  - Loads ASID of queue manager into R2

  - Issues SSAR R2

  - Issues SAR to load x’01’ into ARs, and access data in queue manager address space

Did my WTO’s cause problems?

- Registers 0, 1 14, and 15 updated by the macro.

Tracing backwards from RIT going inactive:

- RITM605S RIT Error recovery routine: RIT now inactive

- Issued from FB_RECOVERY

  - Called from 291 and 728 (re-establishing after MQ call)

  - Just returns in the case of some abends, but not in this case

  - Uses storage area within PC named RECWORK

  - How to tell what happened just prior to recovery routine being invoked?

Does adding PT=YES on ATSET help?

- No, agent still goes inactive at recording start, but no dump?

Have my changes accomplished anything?

- If I remove the ATSET, do I receive an abend again?

Am I copying the wrong ALET to the Ars?

- Updated to use QMALET in Ars, on 9.2 RIT still goes inactive.

- This time I get a dump… 0D7 Reason code 25. This is still indicating that the primary address space is not authorized to issue an SSAR instruction to the target address space. 25 indicates “The S bit is off in the target address space’s AT.”

- How can I view the EAX that is associated with the PC?

Questions

- How can I verify that the SSAR is using my PC’s EAX as an index into the target system’s AT? Will AXEXT do that, or will it simply report the associated AX value?

  - Use TESTART to test “the validity of an ALET and the EAX-authority of the caller to access the address/data space that the ALET represents.”

  - To get the EAX, a program can issue the extract stacked state (ESTA) instruction to retrieve the EAX from the current linkage stack entry.

- Does a non-space switching PC, have its own DU-AL, or does it use the same one that was associated with the TCB that invoked it?

- What could cause an ALESERV ADD with CKKEAX=YES and ACCESS=PRIVATE to execute successfully, but an abend 0D7 RC25 when the SSAR is performed?

After help from Rich

- New abend: 0E0 RC29: An ALET specified an access list entry (ALE) that is not valid.

  - Based on the fact that only AR8 was populated (00010126), I could tell this was in line 2268 of the code.

  - R10 contained 1D42A1C0. QMALET contained 00010126. This is the ALET that I created with ALESERV, so there is something wrong with it.

  - I was still able to start recording without RIT going inactive on 3H. Not sure why?

  - After adding AL=PASN to the ALESERV, I am no longer getting a dump. I cancelled CSQ9RIT with a dump and then browsed the dump to find RITC. Found it at 1F80F970, but it doesn’t look like what I am searching for. Tried ‘F RITC NOBREAK’ and could not find another occurrence of RITC.

  - Should I update the recovery routine to provide more info on the error? Breadcrumb and abend code?

  - Do I have an identifier for the PASN that can be used in a table of ALETs?

    - RITCOM JOBNAME – saved in section L8_DSAOK, which is called by F8_GETDSA, which is called by L_DivertPut and L_ItsAPut  can I also save this in the DSA, then loop through list of jobnames/alets in PCDEF? Or is there a possibility of jobnames not being unique? Should I use ASID rather than job name?

- Why is hrvmmqf being invoked at this point, since it is only refreshing, and is not yet duplicating or diverting? As soon as we refresh, any put is invoking it, and the qmgr’s storage needs to be accessed to find the queue being accessed. This includes puts done internally.

  - Hrvmmqf flow:

    - Check for divert queues, if none, check for duplicate queues

      - If divert queues, find the one that applies, and then LARL F8_GETDSA

    - If no duplicate queues, wto and exit

      - If duplicate queues, LARL F8_GETDSA

      - Find queue name (L_HaveDSA)

        - LARL F9_FindQueue  This includes the second SSAR (line 2198). This is  the one that only uses AR8, which is where we are dying.

      - Determine if queue is in list (L_CheckQueueName, L_findUserKey)

- Hrvmmqf can be invoked from ANY address space. The ALET is associated with either the address space (PASN-AL) or the TCB that called it (DU-AL), therefore, would need a table of ALETs if we want to avoid ALESEREV being called each time hrvmmqf is invoked.

  - If the table is in PCDEF, must ensure that it is only updated by one instance of PCDEF at a time.

  - Is there storage acquired each time hrvmmqf is run? Is that storage freed each time pcdef returns? Yes. If doing an ALESERV ADD each time hrvmmqf is invoked, and removing it each time it ends, it should be safe to save the ALET in the DSA.

  - Should ASID be the key to the table?

    - Some asids are “reusable” and others are “ordinary”. Ordinary asids can be reused when it has either no entry tables, or only entry tables with non-space switch entries. That is our case. “Reusable” asids are always available for reuse. (Coding cross memory services to avoid the loss of ASIDs from reuse - IBM Documentation)

    - The following scenario is possible:

      - Hrvmmqf obtains an alet for the asid to access the qmgr

      - The alet is stored somewhere

      - The address space terminates, causing it’s al to be discarded

      - Something new runs with the same asid

      - The alet for the old address space is found in the table

      - Hrvmmqf attempts to access the qmgr’s storage using the alet, and an abend (0E0?) occurs.

      - Hrvmmqf would then need to issue ALESERV to get a new ALET and update the entry in the table. If this fails, then RIT recording should be disabled with some type of meaningful error message displayed.

  - Should job name be used as the key to the table?

    - Some customers might have multiple jobs with the same name. This poses the same problem as using the asid as the key. However, the condition of finding an outdated alet in the table might happen much more frequently.

- Do ALESERV every time and release ALET every time

  - Move ALESERV to F8_GETDSA (called in line 451, 817) – F7_FindNextQueue called from 437, which is prior to DSA being allocated.

  - Do ALESERV just prior to divert (line 400) and dup processing (line 839) retrieving queue name from the qmgr, and save in R5 until DSA has been allocated. At that point, save to DSA. I can save it in R5

    - Use of AR in F7_FindNextQueue:

      - As of call to F7_FindNextQueue (435), the previous use of R5 was in the end of the setup for FRR

      - The next use of R5 after call to F7_FindNextQueue is in 498, where it is loaded with the address of WTOMSGID.

      - There are no references to R5 within F7_FindNextQueue

      - As of call to F7_findNextQueue(918), the previous use of R5 was to load the address of lQAREA to then subtract it from R7. It is not referenced again until 1049 where it is used to point to the address of WTOMSGID

    - Use of AR in F9_FindQueue

      - As of call to F9_FindQueue in line 400, the previous use of R5 was in the end of the setup for FRR

      - The next use of R5 after call to F9_FindQueue is in 498, where it is loaded with the address of WTOMSGID.

      - Within F9_FindQueue, it is used in 2246, the load of the AR could be done before this, however, we need to keep the ALET somewhere until it can be saved in the DSA. R2 was not used again within F9_FindQueue after using it for AR related stuff, so I can switch the references to R5 from L9_NotPUT1 to use R2.  this has been put back the way it was

      - As of call to F9_FindQueue in 839, the previous use of R5 was in the FRR set up

      - The next use of R5 is to load it with the address of lQAREA and subtract it from R7

    - Use of AR in FA_CopyUserQueueName

      - As of call to FA_CopyUserQueueName in 467, the previous use of R5 was in the end of the setup for FRR

      - The next use of R5 after call to FA_... is in 498, where it is loaded with the address of WTOMSGID.

      - Within FA_... R5 is not used

      - As of call to FA_... in 855, the previous use of R5 was in the FRR set up

      - The next use of R5 is to load it with the address of lQAREA and subtract it from R7

  - Store token within DSA

  - Handle bad return code from ALESERV delete

  - Add ALESERV DELETE at same point when DSA is freed

  - Could I possibly be allocating 2 ALETs using this method?

  - Add notes regarding when R5 is free

  - Tried this and got an 0c4 rc4 on the last instruction of the expanded ALESERV macro.

    - Should I move the ALESERV to the routine that gets the DSA? If I do this, I’ll need to move the call to that routine earlier in the process for diverts, and that would mean acquiring storage for every message, even the ones that don’t get diverted. This would be a performance hit that I would like to avoid if possible.

    - R5 contains 00000000_1CC7CE64 and R14 contains 5 at the time of failure

    - Added parenthesi around R5 in ALET parm, recompiled, reran, and 0C4 went away. Found this by browsing the ALESERV macro.

  - Received abend E0 RC 29. Failing instruction text is 41907006 1B00B25D 0089A784, which is lines 2540 through 2543 in compile listing. Issue appears to be in the CLST R8,R9 in F7_FindReloop. E0/29 is “An ALET specified an access list entry (ALE) that is not valid.” R5 contained D9E3E2E2_000000FC and AR6 and AR8 both contained 000000FC. Could it be that an ALET is being created within both the divert and dup routines, and the one created in the dup routine is invalid since one already exists?

    - Added WTOs after each reference to memory using AR, but got E9 28 return code and no WTOs. DIS ALL after this attempt shows RIT interception is still active, and it shows that RIT.TEST.INPUT3 is being recorded. Dump does not show that RITMQF wsa the current program running.

    - Abend this time was E0/28: “Reserved bit positions in an access list entry token (ALET) were nonzero.”

    - Removed WTOs, except for initial one

    - Started recording of transport. RIT exit was still active.

    - Submitted job to put message, and RIT went inactive. Got dump of E0/29. This is still the CLST R8,R9. The AR contains ‘FC’.

      - From address space analysis within “LINKAGE STACK ENTRY 05 FROM TCB”:

      - EAX 0016

      - SASN and PASN both 29

    - How can I find the AL within the dump? (https://www.ibm.com/docs/en/zos/2.2.0?topic=information-assb-mapping)

      - ASCB offset x150 - 02FE5000

      - ASSBBALV offset 64(x40) - VIRTUAL ADDRESS OF THE BASIC ACCESS LIST

      - ASSBPALV offset 84(x54) - PASN ACCESS LIST - 7FFF7880

    - Am I checking the ALESERV RC? yes

    - What are the contents of QMALET?

    - I am referencing DALESERV in ALESERV, but this is in PCDEF. Is this causing one invocation of hrvmexit to step on another version of it?

    - Will move aleserv to f8_getdsa

      - Recording now works!!!

      - Clean up comments

      - Add error checking to exit if aleserv fails

      - Upload and compile programs needed to test stubbing:

        - MSTUCK.MISC.JCL(COBCMPMQ,MQ91PUTR,MQPUT1CB)

        - MSTUCK.MISC.SOURCE(MQPUTRE2, MQPUT1CB)

      - Test stubbing

        - All looks good after starting the stub, however, RITM605S appears in the job log for the put to INPUT3.

        - Found that recovery routine produces dump after issuing RITM605S

        - 0E0 rc28 in L9_NotPUT1 which is in F9_FindQueue

        - Dump shows 03205BE8 in AR8, which does not look like an ALET. Has R5 been corrupted at this point?

        - Switching from SAR r8,R5 to LAM R8,R8,QMALET resulted in the same problem: R8 contains 03205BE8. Could R13 have become corrupted?

        - Verified that recording still works

        - Moved getdsa to before call to F9_FindQueue. Now there is no 0E0 RC28, but no calls are processed by stub. CSQ9RIT trace output looks normal

        - Moved getdsa to beginning of L_DivertPut

        - Forced dump after getdsa.

          - Both R13 and R15: 1CDCBB10, add 3d8 to find QMALET at 1CDC BEE8…. 01010038. I believe the ALET is a half word, so 0038 is probably correct.

        - Forced dump after F9_Findqueue

          - R15 is 0, causing rest of divert code to be skipped

          - Within F9_Findqueue:

            - R6  1C98D668

            - EBROB is found in 1C98D674 and is 1CA46820

            - ROBACE is found in 1CA46838 and is 1C98D608

            - ACERAL is found in 1C98D630 and is 1CC58768

            - RALMTHR is found in 1CC587B8 and is 5C2BD480

            - RALMTHR is non-zero so is assumed valid

            - After return R2 is non-zero, so assume that QRPL_hObject was positive (R1 contained 1CDCBB00, so QRPL_hObject was at 1CDCBB88… This is 0, however, so perhaps R1 at time of dump had been modified)

        - Forced dump within F9_Findqueue after ‘L     R8,MTHR_HPV(,R8)’ (line2246)

          - No dump was produced, so must have exited F9_Findqueue before this point

        - Forced dump within F9_Findqueue just prior to “L     R2,QRPL_hObject(R1)”

          - R1 at time of error: 1CCA8FF0  This is the last 16 bytes of the DSA

          - R2 is being loaded from 88 past R1 (1CCA 9078) and contains value F0F0F96B, which is a negative number. Why does “Time of error” information show R2 as containing 0000005A? Does R1 contain the wrong value at entry to F9_Findqueue? Earlier in the process, R9 was loaded from R1, and in the dump, R9 contains the same value as R1. So was R1 corrupted prior to that point? Previous reference to R1 was when it was used to load R4 from QRPLCMDR(,R1) which is address 1CCA8FFC and contains 0000005A.

        - Saved R1 before call to GETDSA, and restored it afterwards. Stubbing now works!

      - RC 0 from F9_Findqueue results in jump to L_DoDuplicate, which will cause a second call to getdsa. Not true. Dup code checks for nonzero R13 before obtaining DSA

      - Test PUT1 using MQPUT1CB

        - According to CSQ9RIT log, started recording at 9:27:40. RIT was active at this point.

        - Successfully Recorded 2 messages from CSQ9PUT1 at 9:28:22

        - According to CSQ9RIT log, stopped recording at 9:29:04.

        - According to CSQ9RIT log, started stub at 9:29:39.

        - RITM605S from CSQ9MSTR at 9:29:46

        - Submitted CSQ9PUT1 at 9:29:49

      - Attempts to recreate problem with RITM605S from CSQMSTR

        - MQPUT1 stub:

          - Restarted agent

          - Started MQPUT1 stub at 10:21:52

          - RITM605S at 10:22:15 from CSQ9MSTR

          - Stopped stub

        - First92stub:

          - Restarted agent

          - Started stub at 10:26:12

          - RITM605S at 10:26:15 from CSQ9MSTR

          - Restarted agent

          - Started stub

          - Submitted CSQ9PUT immediately

          - Was able to run CSQ9PUT, CSQ9PUT1 repeatedly and problem did not recur

          - Restarted stub

          - Waited before submitting CSQ9PUT

          - RITM605S at 13:19:51 from CSQ9MSTR

        - Appears to happen when divert rule is in place and message that does not need to be handled by agent is processed prior to message that matches rule. No dump produced today

          - Saved as HRVMMQF5 before making any more changes

          - F9_FindQueue jumps to L9_FindQReturn2 without returning queue name for put1 to queues starting with SYSTEM, AMQ.MQEXPLORER, COM.GREENHAT.COMMAND.QUEUE, and KMQ.

          - Reverted changes made to use R2 in place of R5 within F9_FindQueue. This is no longer needed since I am no longer using R5 to store the ALET. After this, I was unable to reproduce the problem.

          - Saved as HRVMMQF6

      - Test putting messages to queue via MQ explorer

        - Recording worked

        - Creating and running a stub worked

        - Pass through and discard both worked

      - Test transport recording - worked

      - Test with previous version of MQ

        - Stand alone qmgr with security on – recording and stubbing worked

        - Shared queues

- use a special entry-table entry number of 1, corresponding to an EAX of 1

  - HRVMMQF6 and HRVMSAS6 are backups taken before making any of these changes

  - find out how to use entry table entry number 1

    - ETDEF – hrvmsasm

      - updated to use EAX of 1, but received following message: RIT107E Return codes from ETCON 6 0000001c 00000008 00052000 00000319

      - From trace output:

        - PCEAX at offset 1B2 = 0029

        - No rc saved from ETDEF

    - ETCRE – done by srb

      - ETCRE_RC return code FF000000 (preset by us in hrvmsasm)

    - ETCON – done by srb

      - ETCON_RC FF000000

  - Remove ATSET (it follows ETCRE in SRB)

  - Remove AXRES, AXFRE from SRB

  - If unable to use entry-table entry number 1, ensure srb is being scheduled to do AXFRE when agent stops

    - Where does PCDEF get released? – It only gets released if the agent is starting up, finds a token pointing to the old PCDEF, sees that the new PCDEF DSECT is larger than the existing one, and creates a new one. So we can’t simply include code to free the AX whenever PCDEF is released.

    - What gets invoked in these scenarios?

      - Normal stop of agent – branches to STOP to unswap via JAS/BRAS instruction. UNSWAP_Return then issues PR. Assuming my understanding of https://www.ibm.com/docs/en/zos/2.1.0?topic=entry-program-return-pr-instruction is correct, the PR will return control to the next instruction within STOP which then jumps to END_3.

      - Abnormal end of agent (cancel) – Jumps to UNSWAP

      - Stop of qmgr - ???

    - Best to free EAX whenever UNSWAP occurs – tried this, but got 0C4 PIC 4 on ‘ST    R2,ASCHED_RC1’

      - Did something free the DSA before this point? No, it was never obtained when invoked with “STOP” parm.

      - Need to add STORAGE OBTAIN to STOP. What to do if it is not obtained? Write warning message that EAX was not freed?  still getting 0C4 even after storage is obtained??? – most recent abend only occurring after stop command is issued.

        - GOTDSA line 1069 offset A4C

        - ST R2,ASCHED_RC1 line 1324 offset D62

        - R10  7F6235C0

        - ASCHED_RC1 is at offset 2E4 of DSA which would be 7f62 38a4  all 0’s

        - Was obtain successful? What was address of storage obtained? Forced abend immediately after obtain.

          - R15  0

          - R1  1F8B4860

        - Forced abend just prior to USING *.R8 in UNSWAP

          - R1 and R10  1F8B4860

        - Forced abend just after Sched_AXFRE DS 0H

          - R1 and R10  1F8B4860

        - Forced abend just after LA    R2,SCHED_RC1

          - R1 and R10  1F8B4860

          - DATA AT PSW  1F80FB16 - 412060C8  00005020  A2E44120

        - Recompiled without forced abend. Again received 0C4 PIC4. Offset is FB1E, and offset for previous forced abend was FB1C.

          - R1 and R10 have ????

          - DATA AT PSW  1F80FB18 - 60C85020  A2E44120  60CC5020

        - Added SPKA X’80’ prior to STORAGE OBTAIN

          - 0C4 RC4 again

          - Offset FB26

          - DATA AT PSW  1F80FB20 - 60C85020  A2E44120  60CC5020  exact same abend as before.

          - PSW shows execution key 8. What is the key of the storage????????????????????????????????????????? Why does R10 show a different value in the dump????????????????????

            - Where is Sched_AXFRE getting branched from?

              - UNSWAP line 969

                - GOTDSA

                - RESABNORMAL – invoked by RESRECOVERY, which is invoked by the RESMGR. RESMGR is used to intercept cancels. We’re receiving RIT002I in the job log, so I know that RESRECOVERY is being invoked. I temporarily removed the recovery routine and saw that the error was due to having issued a wto with key 8 after the successful AXRES. The bad wto caused a D23 RC 68 abend, which invoked the recovery routine, which then invoked UNSWAP again.

              - RD001 line 985

              - RD_RESTOD

            - Is the fact that I am never setting PCEAX to 0 causing this to get invoked an additional time? – added code to HRVMMQF to reset PCEAX to 0 after successful AXFRE, but this made no difference.

            - Is HRVMSAM getting invoked multiple times for a stop command? No. I added a wto on entrance to HRVMSASM. It was displayed twice when RIT started, but only once when RIT was stopped.

            - Is the RIT888I AXFREE wto causing problems?

      - How can I ensure DSA has been obtained when processing recovery routine? If it has not been obtained, do I need to allocate it before jumping to UNSWAP?

        - It gets obtained before branching to STOP, so I am removing the obtain that I had added in STOP.

        - The problem is that the value of R10 needs to be passed to RESRECOVERY. Currently the values of R6 and R9 are being passed.

        - Updated code to pass value of R6 and R10 to RESRECOVERY, and then obtained the value of R9 from a field (pGlobal) within USERPARM which is pointed to by R6

      - Need to verify that EAX’s are really being cleaned up

        - Getting 0 in R15 on return from IEAMSCH, but this page seems to indicate that R15 only specifies whether the schedule took place, not that the code being executed was successful. IEAMSCHD — Schedule an SRB - IBM Documentation. I can use either the RC in AXFRE_RC or I can test PCEAX to see if it was freed.

        - R9: 1D3FEA00 or 1F8910A0 – no valid data in either location after forcing dump

        - Do I need something like this to prevent LE from corrupting dump?
//CEEOPTS DD *
TERMTHDACT(UADUMP)          this made no difference

        - Do I need to change the SYSMDUMP options?            
CD SET,SYSMDUMP,ALL  This enabled me to see the PCDEF pointed to by R9 in the SYSMDUMP

        - R9  1D3FEA00, AXFRE_RC is at 1D3F EBBC and contains FF000000

        - SRBFREEAX is ‘04’, meaning only the 4 bit is set.TM sets the condition code to 0 if all tested bits are 0.

        - HRVMMQF is testing to see if PCEAX is 0. If so, it skips the AXFRE. Could PCEAX be getting set to 0 somewhere else? 1D3FEBB2 in dump shows PCEAX is 0033 (the next time I ran it showed 0034).

        - SCHED_RC0 is in USERPARM, which should be pointed to by R6 1F864324 + C4 = 1F86 43E8

          - SCHED_R0 00000000

          - SCHED_01 00000000 - SYNCHCOMPADDR

          - SCHED_02 00000012 – SYNCHCODEADDR 12 – SYNCHCOMPADDR Abend code in the same format as field SDWAABCC in the SDWA. But SYNCHCOMPADDR contains all 00’s, so does this really mean anything?

          - SCHED_03 00000000

        - I am receiving the message the “Failed to free” message, but I am not getting the “Failed to schedule SRB” message, telling me that the SRB is getting scheduled, but AXFRE_RC is not getting updated. The scenarios are:

          - Even though I am getting RC 0 in R15, the SRB is not really getting scheduled

            - Comparing this IEAMSCHD with the others:

              - Does R2 need to be zeroed out before the ‘LH R2,PCEAX’?

              - Where is Breadcrumb stored? Can I find that in dump to verify that the dump is from immediately after the IEAMSCHD?

              - In previous IEAMSCHF, the *_RC fields were all initialized with x’FF’ before scheduling

              - In the event that R15 had a non-zero rc after the ieamschd, the existing calls stored R15 in MYRC, and jumped to END_3, but never did anything with MYRC. Maybe LR R5,R15 in END_3 should be L R5,MYRC?

          - The SRB is getting invoked, but the AXFRE section is not being executed. comBreadCrump shows RESLX, SRBFLAG (1B4) is 04 (0000 0100)

            - SRBRESLX 0000 0010

            - SRBRESEAX 0000 0001

            - SRBFREEAX 0000 0100

            - LXRES_RC is FF000000

            - Switching to using JNO after TM made no difference.

            - Writing to combreadcrumb before TM made no difference…. It’s still showing TMRESLX in dump

            - Try dumping after AXRES? SRBFLAG is (1B4) is 01

            - Tried setting SRBFREAX to 00. Still get TMRESLX in dump

            - Tried setting SRBFLAG to SRBRESEAX instead of SRBFREAX to within UNSWAP to see if that is honoured. Even though the dump shows that SRBFLAG contains 01, combreadcrumb contains TMRESLX. This could happen if the srb is not really being scheduled and combreadcrumb just has the residual value from the previous ieamschd. I cleared out conBreadCrumb before scheduling the SRB, and verified in the dump that after the ieamsch, it was still blank. So the error is not with the code within the SRB, it is a problem with the SRB being scheduled. Maybe some of the input parms for ieamschd have not been set up?

              - A_ETCON  stored in DSA, why not in PCDEF?

              - WXTOKEN  stored in DSA, pointer to PCDEF. We also have this in pGlobal. By using pGlobal, we don’t have the same protection against mismatched levels of the code as we do when the agent is starting up. In the case of startup, it obtains the pointer from the name/token, and checks to see if it has changed in size. If it has, it allocates new storage, copies the old to the new, and then releases the old. The lack of this protection in the case of stopping has been there forever.

            - Running with the correct values for A_ETCON and WXTOKEN resulted in the following:

              - R15 0000001C, which means the synch SRB did not complete successfully. See SYNCHCOMPADDR, SYNCHCODEADDR, and SYNCHRSNADDR for more details

              - Dump shows TMFREEAX in combreadcrumb

              - Dump shows PCEAX (1D3FEBB2) contains 0030

              - Dump shows AXFRE_RC (1D3FEBBC) contains FF

              - SCHED_RC0 1F8643E8 RETCODE 0000001C

              - SCHED_RC1 COMP 00000008 – SRB ended abnormally; there is an associated reason code

              - SCHED_RC2 CODE 00052000 – abend code in the same format as field SDWAABCC in the SDWA

              - SCHED_RC3 RSN 00000818 – reason code associated with the abend code. Abend 052 rsn 818 is “An authorization index (AX) may still be in use as an extended authorization index (EAX).”

        - The AXFRE SRB is now failing due to an abend during the AXFREE macro: The caller must ensure that the AXs to be returned are no longer being used by any address space as an AX or an EX; otherwise, the caller abnormally terminates. I see these options to address this. Making backup of HRVMMQF and HRVMSASM to HRVMMQF7 and HRVMSAS7 before attempting these.

          - Skip the AXFREE altogether and only do AXRES if PCEAX is empty. Will this work across qmgr restarts? No

            - Can I record after stop/start of CSQ9RIT? yes

              - PCEAX 1D3FEBB2 - 0032

              - AXRES_RC 1D3FEBB8 - FF

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – 00

              - ETCON_RC EA7C – FF

              - ATSET_RC EBC0 - FF

            - Can I record after cancel/start of CSQ9RIT?

              - PCEAX 1D3FEBB2 -  0032

              - AXRES_RC 1D3FEBB8 - FF

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – 00

              - ETCON_RC EA7C – FF

              - ATSET_RC EBC0 - FF

            - Can I record after stop/start of qmgr while CSQ9RIT up? No. CSQ9RIT failed to start with bad return codes from ETCON.

              - PCEAX 1D3FEBB2 - 0032

              - AXRES_RC 1D3FEBB8 - FF

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – FF

              - ETCON_RC EA7C – FF - RIT107E Return codes from ETCON 6 0000001c 00000008 00052000 00000319 – “The eax was not reserved by the home address space”

              - ATSET_RC EBC0 - FF

          - Skip the AXFREE altogether, and only do AXRES if I see that the QMGR address space has been restarted by checking to see if the qmgr STOKEN has changed. This approach will result in a new EAX each time the qmgr is restarted.

            - Can I record after stop/start of qmgr? Yes

              - PCEAX 1D3FEBB2 – 002E

              - AXRES_RC 1D3FEBB8 - 00

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – 00

              - ETCON_RC EA7C – 00

              - ATSET_RC EBC0 – 00

              - QMSTOKEN EBC8 - 000001D0

              - QMOLDTKN EBD0 - 000001E4

            - Can I record after stop/start of qmgr when RIT up at qmgr stop? yes

              - PCEAX 1D3FEBB2 – 002F

              - AXRES_RC 1D3FEBB8 - 00

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – 00

              - ETCON_RC EA7C – 00

              - ATSET_RC EBC0 – 00

              - QMSTOKEN EBC8 – 1EC

              - QMOLDTKN EBD0 – 1D0

            - Can I record after stop/start of CSQ9RIT? yes

              - PCEAX 1D3FEBB2 – 002F

              - AXRES_RC 1D3FEBB8 - FF

              - LXRES_RC EA84 – FF

              - ETCRE_RC EA80 – 00

              - ETCON_RC EA7C – FF

              - ATSET_RC EBC0 – FF

              - QMSTOKEN EBC8 -1EC

              - QMOLDTKN EBD0 – 1EC

            - Can I record after cancel/start of CSQ9RIT?

              - PCEAX 1D3FEBB2 – 002F

              - AXRES_RC 1D3FEBB8 - FF

              - LXRES_RC EA84 –

              - ETCRE_RC EA80 –

              - ETCON_RC EA7C –

              - ATSET_RC EBC0 – FF

              - QMSTOKEN EBC8 – 1EC

              - QMOLDTKN EBD0 – 1EC

            - Do I ever see the same EAX after qmgr restarts?

              - 0030, 0031, 0032, 0033, 0034  Yes. I have seen 0032 used before, but I’m not sure what causes an old one to be reused vs the next sequential number being used. I checked again the next day, and 0036 and then 0037 were used.

          - Leave code as is and somehow “undefine” the PC. Do I need to do an ETDIS? – will not do this, since the other approach is working.

            - Current flow:

              - Make sure we are authorized

              - Obtain dsa

              - Branch to STARTUP or STOP as appropriate

              - Enqueue

              - Verify HRVMMQF is in LPA and get address

              - Check for valid name token

              - Use existing PCDEF, if we have one, otherwise get storage for a new PCDEF

              - Schedule SRB to reserve an AX

              - Perform ETDEF

              - Schedule SRB to do LXRES, ETCRE, ETCON, and ATSET. We may get abend 052 RC 512. In that case, reuse the LX

              - I should be able to tell whether LXRES completed successfully on last run by checking ETCON_RC, ETCRE_RC, and ATSET_RC. If they are FF, then they weren’t executed.

                - AXRES_RC 1D3FEBB8 - 00

                - LXRES_RC EA84 – FF – only performed if LXVALUE is 0

                - ETCRE_RC EA80 – 00 – Performed even if we already have an LX

                - ETCON_RC EA7C – FF – I assume that the fact that the RC has not been updated means that it failed.

                - ATSET_RC EBC0 – FF – Performed even if we already have an LX

    - Do I need to “un-ATSET” the EAX? No. From the description of AXFRE: On completion of the AXFRE macro, all authorization of the freed AX values in authorization tables for the entire system are purged. Is an AX freed when the home address space associated with the AX ends?

- Need to verify that ALESERV is really deleting ALETs. Updated HRVMMQF to save ALESERV DELETE return code, and verified in a dump that the value is 0.

- Add mechanism to store ALETs

  - How significant is the added overhead? Can we live with it?

  - Create table within PCDEF to maintain list of address spaces and ALETs

    - Each entry should contain an STOKEN and an ALET

    - Should I use compare and swap when updating an entry in the table?

    - Just do round robin

  - Add code to delay obtaining storage for diverts if an ALET already exists for the address space. In this case, in which register would the ALET be saved until storage is obtained?

- SVT testing

  - Queue sharing group – not able to do this until 3H or 3I have been upgraded to at least 2.3. As an alternative, I can at least do regression testing with an earlier version of MQ on 3I.

  - Qmgr with security enabled – need to set up security for CSQ9 before I can do this. As an alternative, I can regression test with an earlier version (QM01) on 3H

  - Provide test fix to customer asap?

- Email Hank - done

- Create defect - done

- What are the migration considerations? Do I need any safeguards in case the new HRVMMQF is used with the old HRVMASM or vice versa?

- Update c programs that access PCDEF

- Clean up code

  - HRVMMQF

    - Remove unneeded comments (especially in areas around where the sar is done)

    - Is L_ALESERV_ERROR ever called? Should it be?

    - Set flag if qmgr has changed. If flag set, check ATSET_RC

    - Either handle AXRES_RC or add comment on why it is not needed

    - Just before the branch to F8_GETDSA in L_DivertPut there is a LR R9,R1, and after there is an LR R1,R9. Is this to preserve R1 contents? Why wasn’t it there before? We needed this, otherwise there was a problem during stubbing.

    - Comment about ALEDEL_RC and ALEADD_RC not being handled… or else handle them. If not handled, remove L_ALESERV_ERROR

    - Put back @VERSION@ and @BUILDLEVEL@

  - HRVMSASM

    - Need new message number to replace RIT888I. Also need to search for other temp messages I have included, and fix their numbers. - done

  - Determine whether eAXRES in RITCOPY is required.

  - Decide whether additions to c program are required for error messages/recovery.

- Test again with 9.2

  - After recording, creating 3 different stubs, one of which had an empty message body, running the stub, sending messages to the stub, stopping the stub, clearing only 2 of the messages from recording studio, and starting recording again, messages were not recorded.

    - Setprog lpa,add done just prior to this at 13:33:12

    - Queue manager STC02486 had been started just 8 minutes before this at 13:33:22

    - Csq9rit started at 13:33:47

    - Recorded csq9put, csq9put1, mq explorer message, stopped recording, then started stub at 13:39:12

    - CSQ9PUT ended at 13:39:26

    - 13:40:22 JOB02493 CSQ9PUT ended

    - 13:41:04 RITM605S RIT Error recovery routine: RIT now inactive – CSQ9MSTR produced SYS1.S0W1.Z23A.DMP00023

    - Abend 0E0 028 - Reserved bit positions in an access list entry token (ALET) were nonzero.

    - PCDEF at 1D3FEA00

    - Failing instruction was “L     R8,MTHR_HPV(,R8)” within L9_NotPUT1. AR8 contained FFFFFFFF, which it loaded from R2.

    - DSA pointed to by R13:  1C79D130, QMALET at offset 3D8: 1c79d508. That location has an F1SA (eyecatcher) followed by almost all 00’s with the exception of 1c79d508 which contains “FFFFFFFF”

    - It’s suspicious that only that one word contained FFFFFFFF, while the other bytes were all 0. But this could be explained by the fact that ALESRV is the first thing that is executed that requires DSA. Just beforer the ALESERV, DSA is cleared and F1SA is moved to ???

    - Can I find the real DSA in the dump?

      - The name of the queue should be in QAREA + 6, and QAREA is pointed to by R7: 1CBCA678 – Not there

    - What is ALEADD_RC? PCDEF+1C4 = 1d3fEBC4  0000000C “The current access list cannot be expanded. There are no free access list entries and the maximum size is reached.”

    - ALEDEL_RC is at PCDEF+1C8 = 1d3febc8  00000000

  - Attempting to recreate without restarting queue manager – could record messages from batch job, but recording message from MQ explorer produced problem immediately, and after that I was unable to record anything.

    - Recreated by only recording

      - Recorded csq9put, csq9put1, and message from mq explorer. Stopped recording.

      - 14:43:23.18 RITM712I Duplicate message OK. From STC2487 CSQ9CHIN

      - 14:43:30.13 RITM605S RIT Error recovery routine: RIT now inactive

    - Unable to recreate when only putting messages from batch job, so the address space whose access list could not be expanded must have been either CSQ9CHIN or CSQ9MSTR

    - Recreated immediately by putting message to queue via MQ explorer, after that, restarting CHINIT and RIT resulted in RIT going inactive within 13 seconds of starting recording.

  - Attempted to recreate after queue manager restart – was unable to recreate

    - Restarted qmgr, then started recording only messages from mq explorer. Problem did not occur.

    - Started CSQ9PUTR, then started recording and putting messages to queue from mq explorer. Problem did not occur

    - Started stub, sent 20 messages from MQ explorer, problem did not occur.

    - Started csq9put stub, sent 10 messages, problem did not occur.

    - Stopped stub, started recording, sent message with csq9put with no data, message from csq9put1, mq explorer, stopped recording, started csq9put stub, sent 4 messages successfully, stopped stub, started recording, deleted 2 messages from recording studio, sent 2 message with no data from csq9put, problem did not occur.

  - Recorded 41230 messages: 41110 from java app, and 120 from CSQPUT9R

  - Recorded 50000 more messages without stopping recording

  - Modified program to put 50000 messages without disconnecting from qm in between. Recorded these messages. DIS ALL shows RIT recording still active and 141110 messages to RIT.TEST.INPUT3 processed and 120 messages to RIT.TEST.OUTPUT3 processed. Recording studio only shows 123655 events, but that may not be related to the agent, since it appears to show the correct number processed.

  - Stopped recording.

  - Recorded 50 messages from java plus replies from CSQ9PUTR

  - Started and stopped recording without recording anything

  - Created stub from 50 messages

  - Sent 50 request messages to stub. Agent shows all 50 messages processed, but stub log only displays messages for 33. Appears to have taken 1 second per message to process.

  - Sent 250 messages to stub. Agent shows all were processed, but stub log only shows 34

  - Restarted stub. Sent 10000 messages to stub. 100 were processed within RIT. DIS ALL shows none were processed, however I see 10000 “Divert message ok” messages in the syslog. Last one was at 07:16:08.63

  - 07:19:39.39 STC02638 RITM605S  issued rom CSQ9MSTR

  - No dumps.

  - Is this repeatable? Does it happen with old agent and MQ V8?

- Perform build and create xmit files

Henry

Another problem during testing yesterday afternoon.

Can’t reproduce, but I’m sure it will happen again

Related to ALET clean up

Move story to system test, defect in next sprint, spike task n sprint 3	  TP-77901